20860-2 tests (2009)

Dielectric withstand voltage--This test, which conforms 60512-4-1, Method A,

Mate two test sample cable assemblies. The SMA connectors on the remaining ends shall be connected in an

adequate way to the test apparatus. The a.c. voltage shall be linearly increased to 500 V and maintained for

60 s. Perform the test at a relative humidity of between 45 % and 75 %.  No dielectric breakdowns or flashover are permitted.

Insulation resistance

7.3.1 Purpose

This test, which conforms to IEC 60512-3-1, Method A,